AutoLens — Mini CANoe Architecture

Problem & Scope

Replace the 3 most-used CANoe features without requiring a dongle:

| Feature | CANoe equivalent | AutoLens module |
| --- | --- | --- |
| Live CAN monitor + DBC decode | Trace Window | TraceWindow |
| Manual + periodic frame sending | IG Block | Generator |
| Log file replay + DBC linking | Replay Block | Simulation |
| Read/Write DIDs + DTCs | UDS Diagnostics | Diagnostics |

Data Sources

Vector VN Hardware (physical)

└─ vxlapi.dll (Vector XL Driver Library — free download, no licence needed)

└─ VectorXLInterface : ICANInterface

DemoInterface : ICANInterface   ← simulated traffic for offline dev/testing

Log Files (.asc, .blf)          ← replay input

DBC Files (.dbc)                ← signal decode database

Key decision: we talk to the DLL via QLibrary (dynamic load), so the app still starts even if the Vector driver isn't installed — it degrades gracefully to Demo mode.

Module Structure

Qt Model Hierarchy

Threading Strategy

Rules:

- Models only mutate on the UI thread (via Qt::QueuedConnection)

- CANWorker never touches models directly — only emits signals

- ISOTP/UDSClient live on UDSThread — request/response via signals

Navigation Structure

Phased Build Plan

What we build new (AutoLens-specific)

DemoCANDriver         — fake traffic generator (dev without HW connected)

AppController         — QML-exposed singleton: owns driver + DBC + all models

TraceModel            — QAbstractTableModel → bound to QML TableView

IGBlockListModel      — QAbstractListModel → IG block list

IGController          — cyclic send logic using QTimer

ReplayPlayer          — .asc log file parser + playback

UDSClient             — ISO 15765-2 CAN-TP + ISO 14229 UDS services

DIDModel              — QAbstractTableModel for DID read/write results

DTCModel              — QAbstractTableModel for DTC list

All QML pages         — TracePage, GeneratorPage, SimulationPage, DiagnosticsPage

Phase 1: Foundation — Trace Window + DBC Decode + Vector HW

Files to create:

- AutoLens/CMakeLists.txt — project setup with CANManager + DBCManager modules

- AutoLens/main.cpp — QML app entry point, register C++ types

- AutoLens/Main.qml — main window with navigation sidebar

- AutoLens/third_party/vector_xl/vxlapi.h — copy from C:/VectorXL/bin/

- AutoLens/src/hardware/CANInterface.h — copy from SPYDER_AutoTrace

- AutoLens/src/hardware/VectorCANDriver.h and .cpp — adapting from the existing driver with updated naming

- AutoLens/src/hardware/DemoCANDriver.h and .cpp — creating a mock driver for testing without hardware

- AutoLens/src/dbc/DBCParser.h — starting the DBC parsing module

- AutoLens/src/dbc/DBCParser.cpp — continuing the parser implementation

- AutoLens/src/trace/TraceModel.h and .cpp — building a table model to display CAN messages

- AutoLens/src/app/AppController.h and .cpp — creating the QML-facing controller that ties everything together

- AutoLens/qml/pages/TracePage.qml, StatusBar.qml, and ChannelSelector.qml — the UI components